ري · Rayy

Solar-powered smart IoT plant that shows its feelings with emoji and sound

Build Guide: Steps in Order and the Purpose of Every File

This guide answers three questions:

1. In which order do we build everything (what comes after what)?

2. How do we do each step, and how do we check it worked?

3. What is every file in the project for, and which ones do we edit?

Graduation project · IoT · 2026

# 1. The big picture

Rayy has four parts plus a solar power part that powers the plant device. They do not talk to each other directly: everything goes through Firebase (the cloud database).

| Part | Folder in the zip | Runs on | What it does |
| --- | --- | --- | --- |
| Plant device | firmware/Rayy/ | ESP32 board (C++, Arduino IDE) | Reads the sensors and the battery/solar voltages, decides the mood, plays melodies, sends data to Firebase |
| Solar power part | docs/11-solar-power.md + firmware/Rayy/power.* | Solar panel + charger + 18650 batteries + 5 V boost | Powers the ESP32 day and night; its battery level is shown in the apps |
| Cloud database | firebase/ + firebase.json | Google Firebase (internet) | Stores the live data, history, diary and settings; checks logins |
| Web dashboard | web/ | Any browser (Firebase Hosting) | Shows the data, charts and diary; changes settings; plays a sound |
| Android app | android/Rayy/ | Android phone (Kotlin) | Same as the web, in a phone app |

Data flow: sensors → ESP32 → (Wi-Fi, every 30 s) → Firebase → web dashboard and Android app (instantly). Settings and the “Play” button go the other way: app → Firebase → ESP32 (within 30 s).

## Why this order?

- Firebase comes before the code, because the ESP32, the web and the app all need the Firebase keys (API key, database URL, accounts).

- The hardware comes before the firmware upload, so you can hear the melodies and see the readings at once.

- The ESP32 comes before the apps, so the apps have real data to show (the web also has a demo mode for earlier testing).

- The solar part comes after everything works on a USB power bank, so a power problem never hides a code problem.

- Calibration and testing come after everything works together.

- AI is future work (docs/12-future-ai.md): the project already stores the history it will need.

# 2. Order of work: what comes after what

| # | Step | Needs first | Result (how to check) | Files you use / edit |
| --- | --- | --- | --- | --- |
| 1 | Buy the components (main + solar part) | — | All items of both tables (see the components Word file) | docs/Rayy-Components-Prices.docx |
| 2 | Unzip the project | — | Folder E:\Rayy-Project (English letters only in the path!) | the zip |
| 3 | Install the software | — | Arduino IDE shows “ESP32 Dev Module”; Android Studio opens | — |
| 4 | Create the Firebase project “Rayy” | Google account | Authentication has 2 users; Realtime Database exists; rules published | firebase/database.rules.json |
| 5 | Register the web app and the Android app in Firebase | Step 4 | You have the firebaseConfig values and google-services.json | — |
| 6 | Wire the hardware on the breadboard | Step 1 | All modules connected (USB unplugged while wiring) | docs/03-wiring.md, docs/images/wiring-diagram.png |
| 7 | Configure and upload the ESP32 firmware | Steps 3, 4, 6 | “hello” melody, Serial Monitor: “signed in to Firebase” | firmware/Rayy/config.h (edit), Rayy.ino (open) |
| 8 | Calibrate the soil sensor | Step 7 | Dry soil ≈ 10–25 %, wet soil ≈ 60–80 % | firmware/Rayy/config.h (edit) |
| 9 | Run and publish the web dashboard | Steps 4, 5 | Dashboard shows the plant’s emoji, badge “Online” | web/firebase-config.js (edit), firebase.json |
| 10 | Build and install the Android app | Steps 3, 5 | App shows live data; “Play” makes the buzzer play | android/Rayy/app/google-services.json (add), Model.kt (check) |
| 11 | Build the solar part | Steps 1, 7 | Apps show 🔋 battery % and “Charging from the sun ☀️” | docs/11-solar-power.md, firmware/Rayy/config.h (SOLAR_ENABLED) |
| 12 | Test everything | Steps 7–11 | All tests in the test table pass | docs/08-testing-calibration.md |
| 13 | Final assembly, report, presentation | Step 12 | Box on the pot, panel in the sun, report written, demo rehearsed | docs/Rayy-Documentation.docx, docs/10-report-outline.md |

Steps 6 (wiring) and 4–5 (Firebase) can be done at the same time by different team members. Steps 9 (web) and 10 (Android) can also be done in parallel once Firebase is ready. The solar part (step 11) can be prepared in parallel but is connected last.

# 3. Step-by-step instructions

## Step 1: Buy the components

Use the separate file Rayy-Components-Prices.docx (Arabic + English names, part codes, quantities, prices, buy links). It has two tables: Part 1 main components (≈ 140–317 SAR) and Part 2 solar components (≈ 121–184 SAR). Important: capacitive soil sensor, passive buzzer, data USB cable, protected 18650 batteries, the 6 V CN3791 charger, and a multimeter.

## Step 2: Unzip the project

- Unzip Rayy-Project.zip directly to a short path with English letters only, for example E:\Rayy-Project.

- Do not use a folder with Arabic letters (like E:\Ray ري): Android Studio refuses to build there.

## Step 3: Install the software

- Arduino IDE 2.x from arduino.cc.

- In Arduino IDE: File → Preferences → Additional boards manager URLs: https://espressif.github.io/arduino-esp32/package_esp32_index.json

- Tools → Board → Boards Manager → install esp32 by Espressif Systems (version 3.x).

- Tools → Manage Libraries → install DHT sensor library (Adafruit), BH1750 (Christopher Laws), ArduinoJson (version 7). Click “Install all” when asked for dependencies.

- USB driver if the board is not detected: CP210x or CH340.

- Android Studio Panda (2025.3.1) or newer.

- Node.js LTS, then in a terminal: npm install -g firebase-tools (to publish the website).

## Step 4: Create the Firebase project

- Open console.firebase.google.com → Add project → name Rayy → you can disable Google Analytics → Create. Firebase creates a unique project ID such as rayy-1a2b3; this is normal.

- Build → Authentication → Get started → Email/Password → Enable.

- Users → Add user: device@rayy.app (for the ESP32) and team@rayy.app (for the web and Android). Choose strong passwords and write them down.

- Build → Realtime Database → Create database → location Belgium (europe-west1) → Start in locked mode. Copy the database URL shown at the top.

- Rules tab → delete everything → paste the content of firebase/database.rules.json → Publish.

## Step 5: Register the apps in Firebase

- Project settings (⚙) → General → Your apps → Web (</>) → nickname Rayy Web → Register → copy the firebaseConfig values.

- Add app → Android → package name `com.rayy.app` (exactly) → nickname Rayy → Register → download `google-services.json`.

- Keep both safely; you need them in steps 9 and 10. The apiKey is also used by the ESP32 in step 7.

## Step 6: Wire the hardware

Unplug the USB cable while wiring. Follow the wiring diagram (docs/images/wiring-diagram.png):

| Module | Connection to the ESP32 |
| --- | --- |
| Soil moisture sensor | VCC → 3V3 · GND → GND · AOUT → GPIO34 |
| DHT22 | + → 3V3 · − → GND · OUT → GPIO4 |
| BH1750 light sensor | VCC → 3V3 · GND → GND · SDA → GPIO21 · SCL → GPIO22 · ADDR → GND |
| Passive buzzer | (+) → 100 Ω → GPIO25 · (−) → GND |
| Push button | one leg → GPIO13 · other leg → GND |
| Power (while building) | 5 V USB power bank / charger → ESP32 USB port (the solar part is added in step 11) |

Check every VCC and GND twice before plugging in the USB cable. A reversed VCC/GND can burn a module.

## Step 7: Configure and upload the ESP32 firmware

- Open firmware/Rayy/Rayy.ino in Arduino IDE (all the other files open as tabs automatically).

- In the config.h tab, fill in your values:

#define WIFI_SSID            "YourWiFi"            // 2.4 GHz Wi-Fi or phone hotspot

#define WIFI_PASSWORD        "YourPassword"

#define FIREBASE_API_KEY     "AIza..."             // apiKey from step 5

#define FIREBASE_DB_URL      "https://rayy-1a2b3-default-rtdb.europe-west1.firebasedatabase.app"

#define FIREBASE_USER_EMAIL  "device@rayy.app"

#define FIREBASE_USER_PASS   "the device password"

#define PLANT_ID             "plant01"

- Tools → Board → ESP32 Dev Module, Tools → Port → the COM port of the board.

- Click Upload (→). If it stays on “Connecting…”, hold the BOOT button on the board.

- Open Tools → Serial Monitor at 115200 baud.

For now keep #define SOLAR_ENABLED 0 in config.h (power bank). You change it to 1 in step 11.

Check: you hear the “hello” melody, the Serial Monitor shows [cloud] signed in to Firebase, and in the Firebase console plants/plant01/live appears and updates every 30 s.

## Step 8: Calibrate the soil sensor

- Hold the sensor in the air → note the raw value in the Serial Monitor (e.g. 3050).

- Put it in a glass of water up to the white line → note the raw value (e.g. 1280).

- Write them in config.h: SOIL_RAW_DRY 3050 and SOIL_RAW_WET 1280 → upload again.

## Step 9: Web dashboard

- Open web/firebase-config.js and paste your firebaseConfig values from step 5.

- Test on your computer: in a terminal inside the web folder run python -m http.server 8000, then open http://localhost:8000 and sign in with team@rayy.app. (Without Firebase settings, http://localhost:8000/?demo=1 shows demo data.)

- Publish: in the project folder run firebase login, then firebase use --add (choose the Rayy project), then firebase deploy --only hosting,database.

Check: https://<project-id>.web.app shows the plant’s emoji (there is no screen on the plant) and the badge says Online.

## Step 10: Android app

- Copy google-services.json (step 5) into android/Rayy/app/.

- Android Studio → File → Open → choose the folder android/Rayy → wait for Gradle sync (first time it downloads Gradle 9.1.0 and the libraries).

- Connect a phone with USB debugging on (or create an emulator) → press ▶ Run.

- APK for any phone: Build → Build App Bundle(s) / APK(s) → Build APK(s) → android/Rayy/app/build/outputs/apk/debug/app-debug.apk.

Check: after signing in with team@rayy.app, the app shows the live emoji and values, and Play makes the buzzer play within 30 s.

The Android project uses Gradle 9.1.0, Android Gradle Plugin 9.0.1 and Kotlin 2.2.21. They run on Java 17 up to Java 25, so the Java of a recent Android Studio works without changing settings. Android Studio must be Panda (2025.3.1) or newer.

## Step 11: Build the solar part

Follow docs/11-solar-power.md and the solar part of the wiring diagram.

| Connection | Details |
| --- | --- |
| Panel → charger | Solar panel (+)/(−) → CN3791 IN+ / IN− |
| Charger → battery | CN3791 BAT+ / BAT− → 2 × 18650 holder (parallel) |
| Battery → boost | Battery (+) → switch → MT3608 VIN+ · battery (−) → VIN− |
| Boost → ESP32 | MT3608 VOUT+ (5.1 V) → ESP32 VIN · VOUT− → GND |
| Battery sensing | Battery (+) → 100 kΩ → GPIO35 → 100 kΩ → GND |
| Panel sensing | Panel (+) → 100 kΩ → 100 kΩ → GPIO39 (VN) → 100 kΩ → GND |

- Connect only the batteries to the MT3608 and turn its small screw until the multimeter shows 5.1 V at VOUT.

- Wire the panel, charger, batteries, switch and the two dividers as in the table.

- In config.h set #define SOLAR_ENABLED 1 and upload with the USB cable. Unplug the USB cable, then connect the MT3608 to VIN/GND and switch on.

- Compare the battery voltage on the multimeter with the Serial Monitor line power: battery=…. If it differs by more than 0.05 V, adjust BATTERY_DIVIDER.

Never connect the USB cable and the MT3608 at the same time. Use protected 18650 cells and keep the charger and batteries inside the box.

Check: in the sun the apps show 🔋 battery % and “Charging from the sun ☀️”; covering the panel shows “Not charging now”.

## Step 12: Test everything

- Pull the soil sensor out → 😫 face + “thirsty” melody + apps show “Thirsty”.

- Put it back in wet soil → 😊 face + “thank you” melody + a diary entry.

- Cover the light sensor during the day → 😞 “needs light”.

- Turn the Wi-Fi off → the melodies still work; the apps show “Offline”.

- Cover the solar panel → the apps show “Not charging now”; leave it outside for 24 h → battery chart rises in the day.

- Full test table: docs/08-testing-calibration.md.

## Step 13: Final assembly, report and presentation

- Move the circuit to a small perfboard or mini breadboard inside a box fixed to the pot, together with the charger, batteries and boost converter; DHT22 and BH1750 outside the box.

- Place the solar panel outside the box, facing the sun. The battery runs the plant ≈ 39 hours without sun.

- Write the report with docs/10-report-outline.md and docs/Rayy-Documentation.docx; follow the 6-week plan in docs/09-project-plan.md.

# 4. What every file is for

Edit? column: Yes = you must put your own values in it · Maybe = only to change behaviour · No = do not change.

## 4.1 Project root

| File | What it does | Edit? |
| --- | --- | --- |
| README.md | Start page: what is in the project, architecture, quick start | No |
| firebase.json | Tells the Firebase CLI to publish the web folder (Hosting) and the rules file (Database) with firebase deploy | No |
| firebase/database.rules.json | Security rules: only signed-in users may read/write; checks values (moisture 0–100, muted true/false) | No (paste in step 4) |

## 4.2 ESP32 firmware: firmware/Rayy/

Open Rayy.ino; Arduino IDE opens all files of the folder as tabs and compiles them together. The folder name must stay Rayy (same as the .ino file).

| File | What it does | Edit? |
| --- | --- | --- |
| Rayy.ino | Main program. setup() starts the sensors, the power monitor, the buzzer and Wi-Fi. loop() reads the sensors and the battery every 2 s, decides the mood, plays melodies, uploads to Firebase every 30 s (2 min in power-saving mode), handles the button | No |
| config.h | All your settings: Wi-Fi name/password, Firebase API key, database URL, device account, PLANT_ID, timing, soil calibration, solar settings (SOLAR_ENABLED, dividers, low-battery level), time zone | Yes |
| pins.h | Which ESP32 pin each module uses (soil 34, battery 35, panel 39, DHT 4, buzzer 25, button 13, I2C 21/22) | Maybe (only if you wire differently) |
| mood.h / mood.cpp | The plant’s “brain”: turns readings into a mood (happy, thirsty, too wet, hot, cold, needs light, sleeping) and decides which melody to play (quiet hours, mute, repeat every 30 min) | Maybe (rules) |
| sensors.h / sensors.cpp | Reads the soil sensor (with calibration → %), DHT22 (temperature, humidity) and BH1750 (lux) | No |
| status.h | The current plant state (readings, power, mood, Wi-Fi) shared with the cloud upload | No |
| power.h / power.cpp | Solar part: reads the battery voltage (GPIO35) and the panel voltage (GPIO39) through the dividers | No |
| power_calc.h | Battery % from voltage, “is charging”, power-saving mode below 15 % (tested on a PC) | No |
| sound.h / sound.cpp | The 9 buzzer melodies (notes + durations), played in the background | Maybe (melodies) |
| cloud.h / cloud.cpp | Wi-Fi connection, clock (NTP, Riyadh time), Firebase login, uploading live / history / events, reading settings and the “Play” command | No |

## 4.3 Web dashboard: web/

| File | What it does | Edit? |
| --- | --- | --- |
| index.html | Page structure: login form, mood card, value cards, chart, diary, “Play” box, settings form | No |
| style.css | Design: colours, layout, Arabic right-to-left, dark mode, phone layout | Maybe (design) |
| app.js | Logic: Firebase login, real-time reading of live/history/events/config, chart (Chart.js), saving settings, “Play” command, Arabic/English texts, demo mode | No |
| firebase-config.js | Your Firebase web keys (from step 5) and PLANT_ID | Yes |

## 4.4 Android app: android/Rayy/

| File | What it does | Edit? |
| --- | --- | --- |
| settings.gradle.kts | Project name Rayy, where to download libraries from, includes the app module | No |
| build.gradle.kts (top) | Versions of the build tools: Android Gradle Plugin 9.0.1, Kotlin 2.2.21, Google Services 4.4.4 | No |
| gradle.properties | Build options (AndroidX, memory, download timeouts, allow non-English path) | No |
| gradle/wrapper/*, gradlew, gradlew.bat | Gradle wrapper: downloads and runs Gradle 9.1.0 automatically | No |
| .gitignore | Files Git should ignore (build output) | No |
| app/build.gradle.kts | App settings: package com.rayy.app, Android versions (min 7.0), Compose, Firebase libraries | No |
| app/google-services.json | Your Firebase Android keys (download in step 5; not in the zip) | Yes (add it) |
| app/src/main/AndroidManifest.xml | App name, icon, Internet permission, start screen | No |
| .../MainActivity.kt | Starts the app: shows the login screen or the main screen | No |
| .../Model.kt | Data classes (live, history, events, settings), reading Firebase data, PLANT_ID and optional DATABASE_URL | Maybe (DATABASE_URL) |
| .../PlantViewModel.kt | Connects to Firebase: login, real-time listeners, “Play” and “Mute” commands | No |
| .../ui/LoginScreen.kt | Email + password screen | No |
| .../ui/MainScreen.kt | Main screen with 3 tabs: Plant (emoji + values + battery/solar + Play/Mute), History (chart incl. battery), Diary | No |
| .../ui/LineChart.kt | Small line chart drawn on a Canvas | No |
| .../ui/Moods.kt | Mood → emoji and texts, melody names, time format | No |
| .../ui/Theme.kt | Green colours, light and dark mode | Maybe (colours) |
| res/values/strings.xml | All English texts of the app | Maybe (texts) |
| res/values-ar/strings.xml | All Arabic texts (used automatically when the phone is in Arabic) | Maybe (texts) |
| res/drawable/ic_launcher.xml | App icon (green plant) | Maybe (icon) |

## 4.5 Tests and documentation

| File | What it does | Edit? |
| --- | --- | --- |
| tests/test_mood.cpp | Automatic tests of the mood logic and the solar battery calculations on a PC (47 checks) | No |
| docs/00-step-by-step-guide.md | Same steps as this guide (Markdown) | No |
| docs/01 … docs/12 (.md) | Chapters: requirements, components, wiring, Firebase, firmware, web, Android, testing, plan, report outline, solar power, future AI | No |
| docs/Rayy-Documentation.docx | All chapters in one Word file, for the graduation report | Maybe |
| docs/Rayy-Components-Prices.docx | Component lists (main part + separate solar part) in Arabic + English with codes, prices and buy links | Maybe (prices) |
| docs/images/* | Wiring diagram, dashboard screenshots, flowcharts | No |

# 5. Values that must match, and where data is stored

## 5.1 Values that must be the same in several files

| Value | Where it must be written | Example |
| --- | --- | --- |
| Plant ID | config.h (PLANT_ID) · web/firebase-config.js (PLANT_ID) · Model.kt (PLANT_ID) | plant01 |
| Database URL | config.h (FIREBASE_DB_URL) · web/firebase-config.js (databaseURL) · inside google-services.json | https://rayy-1a2b3-default-rtdb... |
| Web API key | config.h (FIREBASE_API_KEY) · web/firebase-config.js (apiKey) | AIza... |
| Android package | Firebase Android app · app/build.gradle.kts | com.rayy.app |
| Device account | Firebase Authentication · config.h | device@rayy.app |
| User account | Firebase Authentication · used to sign in to web and Android | team@rayy.app |

## 5.2 Where each part reads and writes in Firebase

| Firebase path | Written by | Read by | Content |
| --- | --- | --- | --- |
| plants/plant01/live | ESP32 (every 30 s) | Web, Android | Latest readings + mood + battery / solar + time |
| plants/plant01/history | ESP32 (every 5 min) | Web, Android (chart) | Readings over time |
| plants/plant01/events | ESP32 (on mood change) | Web, Android (diary) | Mood changes with time |
| plants/plant01/config | Web (settings), Android (mute) | ESP32 (every 30 s) | Thresholds, quiet hours, mute, name |
| plants/plant01/command | Web / Android (“Play”) | ESP32 (then deletes it) | Melody number 1–9 to play |
| plants/plant01/ai | Future AI part (chapter 12) | Web, Android (future) | Reserved: predictions and AI messages |